Here are some templates I made of some of the more commonly used materials using the

Machinist Handbook (25th edition) for material density.  I also changed the color, and lighting

so that the parts look somewhat like the material.  This way you can tell at a glance, of an

assembly, what the materials are.  Also, having the material density set for each component in

an assembly allows you to find the center of mass and other information in the "Mass Properties"

dialog box.

I have also changed the "File, Properties, Custom" by adding a few items like Material,

"Resp. Eng." etc.  these can be made to automatically populate the Title Block in your drawing.

To do this, edit the "Sheet Format" (previously called a template in SW99) in the drawing and

create a note called Material (as an example).  Then link that note to the custom property by right

clicking on the note,then properties, then choose the second button on the right with the icon that looks

like a file folder linked to a sheet of paper.  In that dialogue box check the "External model reference" check

box and then type in the EXACT name of the custom property in this case "Material".  When you close, your

note should have changed from black to blue and look like: $PRPSHEET{Material}

In a similar fashion you can have one or more of these items show up in your bill of materials by adding a column called Material to your bomtemp.xls.  Again this must be spelled exactly the way you spelled it in the custom properties for it to work.  Also, the column must be added between the existing columns.  The left most and right most column must be left in tact.

To make the attached templates show up in the list when you create a new part, perform

the following:

Browse to "C:\Program Files\SolidWorks\data\Templates"  Copy the items in your templates

folder to another location (incase you want them back).  Create the following four folders in the

templates directory:

Part, IN

Part, MM

Assembly

Drawing

Copy the attached files into the appropriate folders.  The folders become the tabs in the templates dialogue

box when you start a new part.

Finally, if you wish to make further modifications to the templates, simply open a new part

as normal choosing the template you wish to modify.  Make the change and then do a file,

save as.  Instead of saving it as a .prt (part file), select .prtdot (part templates) from the pull

down list.

Enjoy

John

=======================================
                             
  John H. Moselage, Applications Engineer
  CAE Northwest (formerly Personal
Workstations)
  Washington Office (Corporate Headquarters)
  18939 120th Ave. NE
  Suite 112
  Bothell, WA 98011

  Phone: (425) 489-9662 Ext. 105
  Fax: (425) 489-0641
  Email: johnm@caenw.com
  Web: www.caenw.com